基线

PS端RAM核数据

耗时0.327ms

E203 跑纯C

===== Benchmark Result =====

Total cycles: 8040143

Total mul num: 282429

Total load num: 639234

Total store num: 66842

Total stall num: 499400

CPU Frequency: 15940321 Hz

Total time: 504 ms

===== Benchmark Done =====

E203+cnn加速器(卷积加速)

===== Benchmark Result =====

Total cycles: 1531035

Total mul num: 829

Total load num: 179109

Total store num: 128768

Total stall num: 866896

CPU Frequency: 15936716 Hz

Total time: 96 ms

===== Benchmark Done =====

纯C基线：504 ms，stall 占比 6.2%，mul 28.2 万次。
e203+cnn加速器：96 ms，加速 5.25 倍，mul 卸载 99.7%，但 store +93%，stall 占比升至 56.6%，平均每次访存 stall 从 0.71 拍升至 2.82 拍。

原因:Stroe增多是因为要往加速器写权重偏置等,stall多是因为走 Mem 总线 → ICB → AXI-Lite → cnn_regs延迟高了,要等.纯C访问的是内部 SRAM / ITCM / DTCM，延迟低
结论：计算卸载成功，瓶颈已从计算转移到 CPU 通过 MMIO 搬运数据。下一步优化聚焦权重预加载与批量传输。

| 指标 | 纯 C | E203+CNN 加速器 | 变化 |
| --- | --- | --- | --- |
| Total cycles | 8,040,143 | 1,531,035 | -81% |
| Total time | 504 ms | 96 ms | -81% |
| mul | 282,429 | 829 | -99.7% |
| load | 639,234 | 179,109 | -72% |
| store | 66,842 | 128,768 | +93% |
| stall | 499,400 | 866,896 | +74% |

| 指标 | 纯 C | 加速器 |
| --- | --- | --- |
| stall / total cycles | 499,400 / 8,040,143 ≈ 6.2% | 866,896 / 1,531,035 ≈ 56.6% |
| 每次访存平均 stall | 499,400 / (639,234+66,842) ≈ 0.71 拍 | 866,896 / (179,109+128,768) ≈ 2.82 拍 |

把权重放到BRAM

===== Benchmark Result =====

Total cycles: 783416

PMU MAC=73514 WLOAD=73514 XLOAD=2108 BUBBLE=4480 TOTAL=82907 AXIWR=7068

Total mul num: 10

Total load num: 24968

Total store num: 18790

Total stall num: 151411

CPU Frequency: 15940321 Hz

Total time: 49 ms

===== Benchmark Done =====

加了多通道池化加速器

===== Benchmark Result =====

Total cycles: 353529

PMU MAC=74634 WLOAD=74634 XLOAD=6588 BUBBLE=4480 TOTAL=86269 AXIWR=12678

Total mul num: 10

Total load num: 22733

Total store num: 20737

Total stall num: 195026

CPU Frequency: 15937372 Hz

Total time: 22 ms

===== Benchmark Done =====

准确率计算

100图测试

#define QUANT_SHIFT   12

===== Batch Result =====

Total:      100

Correct:    8

Accuracy:   8.00%

Avg cycles: 307772

#define QUANT_SHIFT   11

===== Batch Result =====

Total:      100

Correct:    8

Accuracy:   8.00%

Avg cycles: 307822

#define QUANT_SHIFT   10

===== Batch Result =====

Total:      100

Correct:    98

Accuracy:   98.00%

Avg cycles: 307849

#define QUANT_SHIFT   9

===== Batch Result =====

Total:      100

Correct:    100

Accuracy:   100.00%

Avg cycles: 307861

#define QUANT_SHIFT   8

===== Batch Result =====

Total:      100

Correct:    94

Accuracy:   94.00%

Avg cycles: 307855

1000图测试

#define QUANT_SHIFT   9

===== Batch Result =====

Total:      1000

Correct:    989

Accuracy:   98.90%

Avg cycles: 307863

#define QUANT_SHIFT   8

===== Batch Result =====

Total:      1000

Correct:    913

Accuracy:   91.30%

Avg cycles: 307856